# Esp32 Gamepad

# Layout:

## Inputs:

## Directional Keys

## W: Move Forward

## A: Move Left

## S: Move Backward

## D: Move Right

## Abilities

## Q: Focus Enemy

## E: Use Item

## Space: Dodge (single press, cannot be held)

## F: Attack

## Overview:

F

E

Space

D

S

W

A

Q

# Technical Information:

How It Works

- Button Inputs:

  - Each button on the gamepad is connected to a specific pin on the ESP32.

  - When you press a button, the ESP32 detects the change and maps it to a corresponding key (e.g., "W", "A", "Space").

- Input Log on the LCD:

  - The gamepad has an LCD screen that displays the last 5 button inputs, like an "input log."

  - Every key you press is added to this log and shown in real-time, with spaces between the characters for better readability.

  - For example, if you press "W", "A", "S", "D", and "Space," the LCD might display: W A S D _ (where _ represents Space).

- Bluetooth Connection:

  - The ESP32 communicates with your PC or other Bluetooth-enabled devices.

  - Button presses are sent as input signals, which act like keyboard keys. This allows you to control games or other applications.

Limits of the Gamepad

- Maximum Input Display:

  - The LCD can only show the last 5 button presses at any time. Older inputs are removed from the log as new ones are added.

- Key "Space" Behaviour:

  - The "Space" key is unique and cannot be held down like other keys. It is sent as a single input each time you press it.

- No Multi-Key Hold Recognition:

  - The gamepad does not support recognizing two simultaneous key presses. If you press two buttons together, only one is registered at a time.

- Input Timing:

  - There is a slight delay (200 ms) to ensure stable operation. If you rapidly press and release a button, it may not register correctly.

- Bluetooth Range:

  - The gamepad’s Bluetooth connection typically works within a range of about 10 meters. Beyond this, inputs may not be received.

This setup makes the gamepad great for simple games or tasks but limits its use in fast-paced or highly complex games that require precise multi-key inputs or advanced features like analogue controls.

# Contact:

## GitHub:

- https://github.com/YannikSand/Esp32-Gamepad

- Email: MiataIsBack@gmail.com

- This project is open source and I am very open for any kind of improvement

- This project was originally a school Project which turned out to be better than expected

- Tested and mainly used in “DARK SOULS™ II Scholar of the First Sin”.  ( I suck at it)

- You may encounter an Issue with the Port, since you manually have to set it to the correct one in the Python code.

- Have fun testing / using this project!